ISIOLO MUNICIPALITY STORM WATER DRAINAGE WORKS

PHASE I /II

Design Review Document

Final Report

ISIOLO COUNTY

January 2020

CLIENT

COUNTY GOVERNMENT ISIOLO

PO BOX 36 – 30600

ISIOLO

CLIENT

COUNTY GOVERNMENT ISIOLO

PO BOX 36 – 30600

ISIOLO

CONSULTANT

P. O. Box 3166-10140

Nyeri, Kenya.

CONSULTANT

P. O. Box 3166-10140

Nyeri, Kenya.

# EXECUTIVE SUMMERY

Isiolo County is county number 011 according to interim Independent Boundaries Review Commission of Kenya. It is located in the upper eastern region of Kenya. It was established by GoK 2010 which provided for devolution of power and resources as a means of ensuring people are involved in governance and allow better supervision and implementation of policies at the grass root level. Among the functions of the county government under the public works and services include storm water management systems in built areas and water sanitation services.

Isiolo town, the capital administrative capital and the commercial Centre of the county has in the recent past years experienced floods during heavy rains.  The floods impact negatively on the wellbeing of the Isiolo residents and visitors. Damage to properties and in severe flood situations loss of life have been experienced. To mitigate these negative impacts the county Government of Isiolo has embarked on a project to tackle the floods once and all through implementation of an elaborate storm water drainage system implementation.

The county wishes to implement a long-lasting solution to the flood problem within the municipality. Ranks Ltd was contracted to offer consultancy services to review design which had been undertaken by national Water harvesting & Storage Authority (NWHSA) with a view of offering technical direction on remedial measured on the flood problem.

The assignment was divided into two stages as:

- 1(a) short term work phase I (Emergency)

- 1(b) Short Term works phase II

Short term work phase I consists of

- Works to control floods from the Airport side through Kiithe to Merire River.

- Works to contain floods that come along the main Isiolo highway tarmac to join Merire River,

- Improvement of Merire River to its outfall

The following will be key Outputs of phase I

- Training of Merire River and improving its width to 8m at the top and 6m at bed level total length 3.1km

- Construction of five Bridges along Merire River, Isiolo Moyale tarmac road bridge, Ewaso Bridge, Machuguma Bridge, Kanguto and Stage Bridge.

- Implementation of Little Angels Academy-Isiolo Girls Jumia Mosque to Merire River Drain which will be lined total length 3.5km

The Phase I of the project will cost Kshs 87,859,577.40

Phase II of the project will involve

- Improvement of the Isiolo Moyale Road drainage system

- Implementation of the Airport -78 Battalion drainage network

- Implementation of the Ngaremara drainage network

The Phase II of the project will cost 287,591,930.52

The Total cost of this will be Kshs 376,451,507.92 After the full implementation of the proposed works the floods menace in Isiolo town will be contained.

Contents

EXECUTIVE SUMMERY	i

CHAPTER 1: INTRODUCTION	1

1.1	Background	1

1.2.	Location	2

1.3.	Population:	2

1.4.	Climate	2

1.5.	Social-economy	2

1.6.	Catchment Characteristics	3

1.7.	Soils	3

1.8.	Land	3

CHAPTER 2:	PREVIOUS STUDIES	4

2.1.	General	4

2.2.	Isiolo River Basin Integrated Flood Management Plan 2014 by JICA	4

2.3	County Government of Isiolo Flood Disaster Task Force	5

2.3.1	Scope and objectives	5

2.3.2	Task force Action Points and Recommendations	6

1.	Short-term Measures	6

2.	Long-term measures	6

2.3.3	Professional/ Organizational Opinion	6

2.3.4	Consultant findings on the implementation of recommendations	7

2.4	NATIONAL WATER MASTER PLAN 2030- Sectional Report (J) Flood and Drought Disaster  Management	8

CHAPTER 3:  BASELINE SURVEY	11

3.1	Flood Situation in Isiolo town	11

3.1.1	Merire River	11

3.1.2 Isiolo town drainage	11

CHAPTER 4: DESIGN OF ISIOLO STORM WATER DRAINS GENERAL OVERVIEW	13

4.1	Introduction	13

4.2	Location of Public Drainage System	13

4.3	Study site	13

CHAPTER 5 LITTLE ANGELS ACADAMEY – JAMIA MOSQUE – MERIRE RIVER DRAIN	15

5.1 Introduction	15

5.2 Summary of design output	16

5.3 Hydrology	16

5.3.1 Rainfall intensity, mm/hr	17

5.4 	Water conveyance channel design	19

5.4.1 Determining geometric dimension of the channel	19

5.4.2 Determining the best channel through interaction (comparing rectangular and trapezoidal channel)	20

5.5	Conclusion & Recommendation	22

CHAPTER 6: DESIGN OF MERIRE RIVER TRAINING WORKS ON MERIRE RIVER (ISIOLO AIRPORT JN –BULAPESA BRIDGE, 3.1KM)	23

6.1 Introduction	23

6.2 Summary of design output	24

6.3 Hydrology	24

6.3.1 Flood Frequency Analysis	24

6.3.2 Characteristics determining its runoff (Kr).	25

6.4 Design of channel	27

6.4.1 Catchment area	27

6.5 Access Culverts	30

6.6 Conclusion and Recommendation	30

CHAPTER 7: DESIGN OF THE ISIOLO AIRPORT JN- KIITHE ROAD JN STORM WATER DRAIN	31

7.1 Introduction	31

7.2 Summary of design output	31

7.3 Hydrology	31

7.4 Hydraulic Design	35

7.5 Design of Culverts	36

SCHEDULE OF NEW CULVERTS AND THOSE TO BE REHABILITATED	37

7.6 Conclusion and Recommendations	38

CHAPTER 8: DESIGN OF NOTHERN SIDE OF ISIOLO AIRPORT TO 78 BATALLION STORM WATER DRAIN CHANNEL	39

8.1 Introduction	39

8.2 Summary of design output	39

8.4 Design of the Channel.	39

8.4.1 Catchment area	39

8.4.2 Determination of TC, QP and channel sizing	40

8.4 Determination of volumes of cut and fill.	43

8.5 Conclusion and Recommendation	44

CHAPTER 9:  DESIGN OF NGARAMERA FLOOD CONTROL WORKS & NGAREMARA “A”, “B” AND “C’.	45

9.1 Project data	45

9.2 Hydraulics	46

9.3 Catchment area	46

9.3.1 Determination of TC, Qp and Q flowing in each Culvert.	46

9.4 Conclusion and Recommendations	49

CHAPTER 10: DESIGN OF NGAREMARA ‘D”	49

10.1 Catchment area	49

10.2 Determination of TC, QP and Channel sizing	49

10.3 Determination of volumes of cut and fill.	52

10.4 Conclusion and Recommendations	54

ANNEXES

ANNEX 1: SURVEY DATA

# CHAPTER 1: INTRODUCTION

## 1.1	Background

Isiolo County is county number 011 according to interim Independent Boundaries Review Commission of Kenya. It is located in the upper eastern region of Kenya. It was established by GoK 2010 which provided for devolution of power and resources as a means of ensuring people are involved in governance and allow better supervision and implementation of policies at the grass root level. Among the functions of the county government under the public works and services include storm water management systems in built areas and water sanitation services.

Isiolo town, the capital administrative capital and the commercial Centre of the county has in the recent past years frequently been experiencing floods during heavy rains. The floods have been hazardous, characterized by destruction of businesses, properties and lives. In addition, due to inadequate drains in the town diseases outbreaks like cholera also.

The county wishes to implement a long-lasting solution to the flood problem within the municipality. Ranks Ltd was contracted to offer consultancy services to review design which had been undertaken by national Water harvesting & Storage Authority (NWHSA) with a view of offering technical direction on remedial on the flood problem.

The assignment was divided into two stages as:

- 1(a) short term work phase I (Emergency)

- 1(b) Short Term works phase II

Short term work phase I consists of

- Works to control floods from the Airport side through Kiithe to Merire River.

- Works to contain floods that come along the main Isiolo highway tarmac to join Merire River,

- Improvement of Merire River to its outfall

The following will be key Outputs of phase I

- Training of Merire River and improving its width to 8m at the top and 6m at bed level total length 3.1km

- Construction of five Bridges along Merire River, Isiolo Moyale tarmac road bridge, Ewaso Bridge, Machuguma Bridge, Kanguto and Stage Bridge.

- Implementation of Little Angels Academy-Isiolo Girls Jumia Mosque to Merire River Drain which will be lined total length 2.1km

The Phase I of the project will cost Kshs 87,859,577.40

Phase II of the project will involve

- Improvement of the Isiolo Moyale Road drainage system

- Implementation of the Airport -78 Battalion drainage network

- Implementation of the Ngaremara drainage network

The Phase II of the project will cost 288,591,930.52

The Total cost of this will be Kshs 376,451,507.92 After the full implementation of the proposed works the floods menace in Isiolo town will be contained.

## 1.2.	Location

Isiolo is one of the 47 counties which is located in the former Eastern province. It is bordered by 7 counties. Tana river to the south, Garissa County to the west, Wajir County to the North West, Marsabit to the North East, Samburu County to the East and Laikipia county to the South East and Meru County to the South West. It lies between GPS coordinates 015N0 30N and 37 30S 37 45 0E

Map of Kenya				               Map of Isiolo County

It covers an area of 25,336km2 and has three sub-counties; Isiolo, Merti and Garbatula.

## 1.3.	Population:

According to the 2009 Kenya population and housing Census, the population stands at 143,294 with a population density of 5.66 people per KM2 and an annual growth rate of 1.45%. The age distribution is; 0-14 years at 44%, 15-64 years at 52% and 65 years and above at 4%.

Isiolo town is the administrative capital of the county and geographically is located right at the center of Kenya.

## 1.4.	Climate

Temperatures range from a minimum of 1200C to a maximum of 2800C. Rainfall ranges from 150mm to 650 mm per annum typical of ASAL areas.

## 1.5.	Social-economy

Main economic activities include pastoralism, subsistence agriculture, small- scale trade and limited harvesting of gum Arabica resin. They also derive their livelihood from entertaining tourists who visits seasonally to watch the Isiolo big five.

The main agricultural products are beef, milk and limited cultivation of crops.

Natural resources are forests, wildlife, building sand, water, pasture, land and potential for mining of; graphite, sapphire, feldspar, muscovite, kaolin, beryl, copper, corundum, garnet, bismuth, silica, iron ore and pyrite.

The LAPSSET corridor which seeks to link Lamu Port in Kenya to Ethiopia and South Sudan with Isiolo at the center and the construction of the Isiolo International Airport and Isiolo Resort City is set to open up Isiolo County. Already, the building of the 501 km Isiolo- Moyale highway, which link Kenya to Ethiopia, is so far complete and the Isiolo abattoir is almost at completion.

## 1.6.	Catchment Characteristics

Isiolo town lies on the downstream of Meru county and foot of Mount Kenya which is situated at a height of approximately 1500mASL. O the other hand Isiolo town is generally very flat rising from elevation 1040mASL and 1160mASL. Consequently, water flows at a very high velocity to a very low lying Isiolo Town which is characterized by inadequate storm drains to evacuate the floods.

Isiolo River has an approximate total length of 95Km running from South to North and a catchment of 683km2. It has many tributaries with major tributaries being Western Marania which changes its name downwards to Lewa and Kandagochi before joining Isiolo and Eastern Marania with a major tributary being Merire which passes through Isiolo Town.

Isiolo river Basin and Elevation Distribution

## 1.7.	Soils

Soils were investigated along the route where the works are proposed to be undertaken through trial pitting. The trial pits were dug to a depth of at most 1.5m. Most of the pits were characterized by blackish- brownish clayey loamy sols which do not drain easily.

## 1.8.	Land

Land is privately owned in Isiolo but the biggest challenge is that most parcels of land lack titles. This problem has persisted for many years and the current government is pursuing to have members of the community acquires titles.

# CHAPTER 2: PREVIOUS STUDIES

## 2.1.	General

Due to the magnitude and cyclic nature of floods that occur and cause damage to life and property, there has been past studies initiated with objective of finding a solution to the flooding problem. Such studies include study by National Water Master Plan 2030, JICA in 2014 and later study by Isiolo Flood Disaster Taskforce. However, these studies identify the flood problems and causes but none of the studies has gone further to quantify the work required to address the flood problem in a form which can enable the works to be implemented.

## 2.2.	Isiolo River Basin Integrated Flood Management Plan 2014 by JICA

JICA engaged by WARMA undertook a detailed study of the flooding situation in Isiolo town with a view of coming up with an integrated flood management plan focusing on both structural and non-structural measures. The study focused on the hydrology of Isiolo river basin from the source to its terminating point in relation to flooding.

The report identified two types of floods that have occurred in Isiolo and categorized them according to damage that they have caused. Food type A being one which cause inundation of the urban area and type B being that which cases inundation caused by overflow and dyke break. The recent floods and damages that have occurred were recorded as:

JICA correlated flood damage with natural condition and social & economic condition in a local area. Natural condition defined types of hazards in a river basin and the social & economic condition defined vulnerabilities and exposures. Characteristics of flood damages in Isiolo town were therefore analyzed bearing in mind the two concepts as follows:

Flood characteristic from Natural conditions

- Topographical gradient is 1/50 and river bed gradient is 1/100 of Merire River, therefore, flood arrival time is short and surface is inundated with water flowing at a very high velocity.

- Rain water doesn’t seep underground and surface run-off occurs in the basin because surface soil is chiefly composed of Planosols with a semi permeable murram layer at 40cm depth

Characteristics of Flood Damage from Socio-economic Conditions

- Population in the Isiolo town area is approximately 60,000 and therefore, flood damage results to high impact to social-economic in the region.

- Infrastructure development projects such as expansion of the airport, construction of roads and housing in progress according to the regional master plan do not take consideration of impacts of flood management.

Flood Damage Mechanism

- Lack of flow capacity in the Merire River (River Water)

The river has inadequate cross-sectional width and depth to carry discharges. In additional, downstream of the Merire River, houses are built in the riparian land illegally. Consequently, it is very difficult to widen the river channel. Furthermore, sediment runoff occurs in the upstream of the Merire River and accumulates in the downstream. Besides the accumulation of sediment, there is also clogging of culverts by illegal dumping of garbage as one of the causes of reduction of flow capacity.

- Poor storm drainage system in whole Isiolo town (Inland Water)

Drainage system such as drainage channels, Calvert, side ditches of road etc. have not been developed to function as a network in Isiolo town. Consequently, during heavy rain the water flood into roads, business premises and residences along the roads.

- Changing water course due to construction of airport and roads

In recent years, due to construction of airport and road in the higher altitude areas of Isiolo town, water course changed due to the new facilities. In the past, rain water flowing from southern part of urban area didn’t flow in a direction to the town area, it travelled down to the north in the eastern slope of the town. After the construction of those facilities, rain water from the south is blocked by road embankment from the airport road and it is led to the town area.

## 2.3	County Government of Isiolo Flood Disaster Task Force

A directive issued by Isiolo county Governor led to formation of a task force to look in to the causes of the flush floods. The task force comprised of Chief Officers from Energy Environment and Climate change, Public Works and Urban Development, Lands and Physical Planning, Roads and Infrastructure, Water and Natural Resources, Town Administrator Isiolo County, Director of Environment, ACC Isiolo Central, KENHA, WARMA Isiolo, County Director Meteorological Services, Environment Officer NEMA, County legal Officer, County Surveyor Isiolo, KENHA (Nairobi to Moyale) and NCA Upper Eastern Isiolo.

## 2.3.1	Scope and objectives

The scope of the assignment was to study the causes, effects and mitigation of storm water within urban areas of Isiolo town. The specific objectives included:

- Identify causes of the flush floods.

- Identify obstructions on the waterways.

- Identification of riparian reserve.

- Professional opinion from MDAs on the cause of the flood.

- Come up with the recommendations and the way forward.

The taskforce observed that causes of floods in Isiolo town were attributed to:

- Lack of flow capacity of Merire River

- Changing water course due to development projects- roads and airport

- Poor Storm Drainage System in Isiolo Town

## 2.3.2	Task force Action Points and Recommendations

### 1.	Short-term Measures

It was agreed that every institution/MDAs take immediate action within their mandates and powers to save the situation. This included clearing of the water channels, alignment of drainage systems, surveying of the area plots to establish encroachments and identification of ownership and serving of notices to those within riparian reserve. In this regard, the following tasks were shared with immediate implementation by the members:

- Lands and Physical Planning department

The taskforce tasked the department to identify the rightful owners with land on the riparian of Merire River since they could only get search for one parcel and they promised to get the rest from Nairobi. It was agreed that the department provide the names of the owners and share with NEMA and WARMA for notices.

- Water Resources Management Authority (WARMA)

WARMA and NEMA were tasked with the responsibility of identifying the riparian reserves along the affected areas and further jointly serve notices to any occupant n the riparian area.

- Kenya National Highway Authority (KeNHA)

Complains were raised by the community in regards to KeNHA culverts around the affected area specially the three culverts along the section of Airport junction and Oil Libya. The basis of the complaints was both on sizes and structure of culverts. The residents explained that there were larger culverts which were well established and faced the direction of water flow unlike the current ones which are small and are not angled to receive the waters therefore structurally defective.

The taskforce tasked KeNHA to do the necessary corrections as soon as possible. KeNHA team requested for official communication from the county government especially with consequences of more waters downstream due to bigger culverts. The Department of Public Works and Urban development was tasked to do the communication through the office of Town Administrator. KeNHA team immediately engaged their contractor on clearing the water channels and promised to do the channel alignment as requested.

- National Environmental Management Authority (NEMA)

NEMA was tasked to provide EIAs for major development projects and especially the Airport which was deemed to be largely contributing to flooding in Isiolo town. The report provided was not satisfactory as it was only for an upgrade of an air strip with no proper mitigation measures. NEMA was therefore tasked with the responsibility of initiating new EIA report for the Airport.

### 2.	Long-term measures

The county Government and other stakeholders to implement Isiolo River Basin Integrated Flood Management Plan.

## 2.3.3	Professional/ Organizational Opinion

WARMA recommendations

- County government to evaluate the Flood Management Plan and initiate implementation of recommendations.

- Opening of the water courses within the town to allow flow of flood waters in the town.

- Opening of the blocked Merire water course to enable flood water from upstream takes its natural courses.

- Riparian area along Merire water course to be identified and any obstruction removed.

- Harvesting and storage of run off in the upper zone of the Isiolo River sub catchment for multiple water use.

- Development of adequate soil and water conservation structures in the upper zone to deal with runoff that finds its way to Isiolo Town.

- Support Isiolo WRUA to implement catchment management and storm water conservation activities contained in the WRUA SCMP.

NCA Specifications

- County to share development plans through the Physical Planner with NCA. This is to ensure that no inadvertent approval is given for structures on waterways.

- NCA to be on board for approval at the county to ensure any concerns pertaining to riparian’s are on board at an early stage.

- KENHA to share development plans for the emergency mitigation on the roads with NCA with a view to ensuring the latter assists in quality assurance during implementation.

KeNHA Solution Input Required

- Construction of cross box culvert at critical flood water crossing points.

- The drains to be lined.

- Stone pitching to the road shoulder.

- Identify and remove all encroachments on the road reserve.

## 2.3.4	Consultant findings on the implementation of recommendations

It was reported that none of the agency had implemented the recommendations apart from KeNHA who had cleared the culverts through their regular road maintenance however at the time of study some were clogged by sediment or dumped garbage on the river bed. See the pictures below.

Clogged Culverts in Isiolo Town

## 2.4 NATIONAL WATER MASTER PLAN 2030- Sectional Report (J) Flood and Drought Disaster Management

NWMP (2030) was launched in 2013 and sectional plan J covers pre-disaster stage issues of flood and draught disaster management in the country. Post-disaster related activities such as food provision, disease prevention, etc., have been basically dealt with by sectors and other than the water sector. These sectors usually include National Disaster Operation Centre (NDOC) and Crises Response Centre (CRC). The fundamental structure of disaster management components in NWMP 2030’

The sectional plan J covers the seven water basins in the country, however in this report only flood related concerns raised on Isiolo are highlighted.

Characteristics of Flood and Damage

According to the Flood Mitigation Strategy (2009), floods occur in Kenya due to natural factors such as flash floods, river floods and costal floods. Records of flood disasters indicate that the worst floods occurred in the period of 1961-1962 and 1997-1998, and latter ones being the most intense, most widespread and most severe.

According to the Catchment Management Strategy and NWCPC, the specific areas that experience floods almost annually include:

- Nyanza Province (Kano plains, Nyakach area, Rachuonyo/Sondu and Migori/Gucha),

- Western Province (Budalangi),

- Coastal Province (Kilifi, Kwale and Tana river Basin),

- North Eastern Province (Garissa, Wajir, Ijara and Mandera),

- Urban Centre (Nairobi, Nakuru, Mombasa and Kisumu),

- Tana River District (Lower Parts),

- Eastern Province (Isiolo), and

- Rift valley (Mogotio, Turkana and Narok).

Ewaso Ng’iro North Catchment Area

Flood Situation

Although most of ENNCA is regarded as an arid district, severe flood damages have been reported in various parts of the catchment area. In 2005, the Daua River which flows along the Kenya – Ethiopia boarder, burst its banks which destroyed many farms in the ramu area of Mandera district. During the short rainy season in 2006, the Daua River overflowed rapidly and caused wide range of inundation in several divisions of Mandera district. The flood that occurred in 2009 caused 16 deaths in Mandera.

In urban area of Isiolo town, flood is frequently caused by the small channel flowing in to the Isiolo River, the tributary of the Ewaso Ng’iro North River, due to lack of discharge capacity of the channel.

Existing Plans

NWCPC Strategic Plan

The NWCPC Strategic Plan 2010-2015 contains the following plans for study and implementation of flood management structure projects. The plan aims to achieve one of its strategic objectives which is to reduce the risks of (or mitigate the effects of) floods:

- Construct five flood management projects, mainly the Nyando, Nzoia, lumi, Tana and Daua River flood management project (based on the study on causes and effects of floods in Nyanza and Western Provinces, Tana Basin and Taita-Taveta District).

- Study and construct six flood management projects, namely the Narok. Turkana, Mogotio. River Gucha, Isiolo and Sabaki-Galana flood management projects.

Flood Disaster Management Plan for Ewaso Ng’iro North Catchment Area

I	Management Strategy

Isiolo has both rivers induced flood and Urban drainage issues. In consideration of the high population density of Isiolo (46000, 2009), this area should be protected by river structural measures. In addition, it is more effective to adopt a strategy to mitigate human damages since structural measures alone have limited safety against extraordinary floods exceeding their design level. A hazard map and an evacuation plan should be prepared. Regarding urban drainage issues in Isiolo, the drainage system should also be improved in consideration of the high population density.

The following are the basic policies in formulating the flood disaster management plan in Isiolo

- Implementation of flood control measures as well as preparation of hazard map and evacuation plan in Isiolo.

- Implementation of urban drainage measures in Isiolo.

II	Proposed Flood Disaster Management Plan

Implementation of flood Control Measures in Isiolo

For the flood control measures in Isiolo, it will be effective to implement a measure to increase the discharge capacity of small channels flowing through the built-up area of Isiolo into the tributary of Ewaso Ng’iro North River.

- River improvement works alone:

Construction of new dyke, reinforcing or heightening of existing dyke, widening of high-water channel by realignment of existing dyke, widening of low water channel by excavation, etc.

- Flood discharge control by several small-scale dams/pans + river improvement works

Adopting natural retarding effects and a flood hazard map and an evacuation plan shall be prepared in the same

Implementation of Urban Drainage Measures in Isiolo

It was proposed to implement urban drainage measures works in Isiolo would be the responsibility of local Authorities, namely, the Isiolo County and the Isiolo Urban Centre. The preliminarily estimated costs of the drainage work composed of gravity drains based on NWMP (1992) were 155.34 Million and 30 for implementation of non-structural measures. However, it is to be noted that drainage works involves, in some cases, major associated works such as pumping station, retarding basin, improvement of receiving river channels, etc., which should be planned in detail in the future.

Recent floods in Kenya

| Date/Year | Area | Type/Main Cause | Number of Damages Catchment |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Killed | Affected Population | Affected Households |
| Mar 2010 | Garfasa in Garbatula in Isiolo | Floods |  |  | 200 |
| Dec 2009- Jan 2010 | Isiolo | Floods |  |  | 7 |
| 2006 | Isiolo | Floods |  | 3,000 |  |
| 23 Oct 2006 | Coast, North-Eastern, Western, and Rift Valley Provinces- Isiolo (Ngarmara, Malkagala, Merti, Gafarsa). Garissa (Hagadera, Ahantabak, Alikune, Jarirot Amuma,Boralgi, Daadab,Ifo). Tana River District (Bula Bahati, Mnazini, Witu, Hola, Through Wenje to Garsen). Mandera (ElWak). Wajir (Guarar, Dajabula, Kursin). Kilifi, Mombasa, Kwale,Kilifi. Lodwar, Moyale (Bori). Ijara. Merti division. Machakos. Modogashe. Mwingi. Kwale. Nakuru. Nyando. Kisumu (Kajulu, Migori, Nyando), Busia (Budalangi,Maduma, South Bunyala), Lugari, Keiyo (Epke). Malindi, Kisumu, Nyanza area | Heavy rain | 150 | 700,000 |  |
| Oct 15, 2006 | Isiolo, Garissa, Lodwar | General Flood | 30 | 30,000 |  |
| April 4, 2006 | Nyanza, Western, Rift valley, Coast and part of North Eastern provinces – districts: Nyando (Kabonyo, Kakola, Kochogo, Ongeche), Rachuonyo (Kayitir, Kawadhgone, Koyugi), Nyatike (Kaden), Migori, Garissa (Dadaab), Isiolo (Merti, Gulesa and Malkagala), Karachuonyo, Homa Bay (Kochia, West Kagan, Rangwe), Ijara, Tana River. Kisumu (Buoye, Winam, Kolwa). Kochia. Naivasha, Nakuru | Heavy Rain | 4 | 40,000 |  |

# CHAPTER 3:  BASELINE SURVEY

NWCPC team undertook to verify the information from past records with the stakeholders and the community members and established the records to be factual representation of the flood situation on the ground.

## 3.1	Flood Situation in Isiolo town

### 3.1.1	Merire River

Merire River is a dry river and a tributary of Isiolo River. It passes through Isiolo town from Meru hills to Isiolo River.

The river course has been heavily encroached with permanent developments and its walls were observed to have been destroyed by water in the past near Isiolo Airport junction. The river runs through Kula Mawe Estate, Bullapesa slum, Safi Estate, Ollabule Estate & slaughter house area. Class D earth roads connect these estates and generally do not have developed drains.

The river is marked with garbage and stagnating water especially near the prison where it was reported that the prison farm has blocked the stream to impound water for irrigation.

A series of 900mm diameter pipe cross culvert adjoin the opposite sides of the drains acting as a drift but because of damping in the drains and sediment run-off they are characteristically mostly blocked. It was reported that the blockages are usually the sources of flood.

### 3.1.2 Isiolo town drainage

Nairobi-Isiolo- Marsabit A2 runs through Isiolo town. It is managed and maintained by KeNHA. The road embankment is generally raised above a normal geographical topography of the town and therefore it is naturally draining. However, only short section of the road has the drainage been provided.

Most the other roads connecting to the road are earth roads and lack storm water drainage facilities.

Isiolo international Airport, a very recent infrastructure development sits on the upstream Eastern side of Isiolo town. It covers a very expansive land with runways, offices and undeveloped land. The institution has provided drains within its facility up to their fences where it is released to the roads adjoining the town. Consequently, all the waters from the Airport is directly drained in to town with no drainage causing flush floods.

The water collects at Merire River which connect to Isiolo River at downstream

# CHAPTER 4: DESIGN OF ISIOLO STORM WATER DRAINS GENERAL OVERVIEW

## 4.1	Introduction

The objective of urban storm drainage is to optimize safe passage of vehicular traffic by collecting storm-water from the road way, and to convey it safely to an adequate receiving body without undue risk to pedestrian traffic or contributing to damage of adjacent private properties during the design storm event.

The flow of water along the roadway can interfere with or halt highway traffic. The most destructive effects of an inadequate drainage system are damage to surrounding or adjacent properties, deterioration of the roadway components, and hazard or delay to traffic caused by excessive pounding in sags or excessive flow along roadway grades.

A storm-water drainage system should be designed to collect and convey run-off generated within a catchment area during and after events, for safe discharge into a receiving water course or the sea. The magnitude of peak flows that have to be accommodated will depend primarily on the intensity of rainfall and the size, topography, soil type, configuration and land use of the catchment.

## 4.2	Location of Public Drainage System

As far as possible, storm-water drainage systems should be located on Government Land and all drains, culverts and pipelines should be located either in road reserves or specially designated drainage reserves, where are non-building areas. Such reserves are essential in order to ensure that there is free and unrestricted access at all times for construction, repair and maintenance.

Drainage reserves should be included where necessary on the various statutory and non-statutory town plans. The width of the reserve should be determined from the requirements for working space, vehicular access for construction plant, depth of the storm-water drain and clearance from adjacent existing structures and foundations. In general, a minimum width of 6 m plus the outside diameter of the pipeline or outside width of culvert is recommended.

## 4.3	Study site

The storm water design is conceptualized to drain a catchment of 2.12Km2 stretching from Isiolo International Airport to the exit of Merire River downstream of Bullapesa Bridge. The catchment is characterized by residential area, shopping area, the Airport run-ways, Earth roads, expansive grass covered airport land.

Ranks Ltd team in consultation with the stakeholders prioritized structural works as proposed in various reports and available funds for construction and the choice of the design works is proposed to be:

- Development of a network Isiolo storm water drainage.

- Deepening and widening for Merire River.

Resources and time provided for  review ,  survey and the design of the works was a limitation to the field survey works that could be conducted and therefore, the team in consultation with the stakeholders focused on the heavily affected route of storm water which was identified as little angels Academy-Isiolo girls – Kiithe- Merire River.

The proposed short term works for the draining Isiolo include:

The assignment was divided into two stages as:

- 1(a) short term work phase I (Emergency)

- 1(b) Short Term works phase II

Short term work phase I consists of

- Works to control floods from the Airport side through Kiithe to Merire River.

- Works to contain floods that come along the main Isiolo highway tarmac to join Merire River,

- Improvement of Merire River to its outfall

The following will be key Outputs of phase I

- Training of Merire River and improving its width to 8m at the top and 6m at bed level total length 3.1km

- Construction of five Bridges along Merire River, Isiolo Moyale tarmac road bridge, Ewaso Bridge, Machuguma Bridge, Kanguto and Stage Bridge.

- Implementation of Little Angels Academy-Isiolo Girls Jumia Mosque to Merire River Drain which will be lined total length 2.1km

In the following chapters the components of the drainage networks are separately considered.

# CHAPTER 5 LITTLE ANGELS ACADAMEY – JAMIA MOSQUE – MERIRE RIVER DRAIN

## 5.1 Introduction

It was observed that the Isiolo Airport, a recent major infrastructure contributing to floods in Isiolo town, has 2 main drainage channels which exit at Little Angels Academy at GPS coordinates 0020’16.32”N 37035’12.20”E and Isiolo girls Secondary at GPS coordinates 0020’43.37”N 37035’12.98”E. once the water leaves the Airport it is released on the existing earth road which is undeveloped and that lacks a storm drainage.

Flood gates along Isiolo Int Airport fence                     Gulley along Little Angels Academy – Isiolo Girls Secondary School Road

The proposed drain is approximately 2.3Km long, it runs along undeveloped area Little Angels Academy – Kiithe road and crosses Nairobi – Isiolo – Moyale A2 road at Jumia Mosque through 2 no., 900mm Dia existing pipe culverts which drain the road towards Merire river.

The road from Little Angels Academy – Isiolo Girls Secondary School – Merire drain is managed and maintained by Kenya Rural Roads Authority (KeRRA). The section from Little Angels Academy - Isiolo Girls Secondary School is approximately 1200m and is undeveloped and non-motorable since storm water has formed gulley’s at the center of the road. The section from Jumia Mosque to Merire drain is approximately 200m and seems to have been a murram road which has since been damaged by the passage of water forming gulley’s right at the center.

Several utilities are in the road reserves like water supply lines ranging from to 325mm -90mm dia, sewer lines of 150mm dia and electricity lines. Developments like schools, shops and hotels also characterize the route.

The works comprises of but not limited to:

- Excavation of the channel.

- Construction of the channel.

- Construction of pipe culverts at specific locations along the channel.

## 5.2 Summary of design output

| Channel | Little Angels Academy - Jumia Mosque - Merire River Storm Water Drain |
| --- | --- |
| County | Isiolo |
| Sub-county | Isiolo |
| Length, km | 2.22 |
| Type of channel | Trapezoidal Channel & Rectangular Covered Channel |
| Channel Dimensions | Top width = 2.55m, Bottom width =.75m, side slopes 1:1, 0.9m Varying Height with 0.3m free board |
| Culvert type | Double pipe precast culvert each of Diameter 1.2m |
| Design Catchment, Km2 | 2.12 |
| Discharge, Q, m3/s | 1.69 |
| Flood return Period | 1 in 50 years |

## 5.3 Hydrology

Rational method is used in the determination of design flood in accordance to the consequences of the flood return period of 10 years. The design flood is used in the sizing of the system. Due to heavy motorist and pedestrian use of the proposed route, the most practical channel is a covered open channel system along Kiithe road to Merire River and an open channel from Little Angels Academy to Isiolo Girls.

The rational method is based on the assumptions that;

- The predicted peak discharge has the same probability of occurrence (Return period) as the used rainfall intensity.

- The runoff coefficient is constant during the rain storm.

The formula was used to determine the design run-off discharge that shall be carried by the proposed drain.

Q=CIA/3.6= I (∑CA)/3.6

Where Q = Peak run-off rate, m3/s

C = run-off coefficient, dimensionless

I = rainfall intensity, mm/hr

A = drainage area, Km2

Run-off coefficient, dimensionless

Dimensionless value representing characteristics of the watershed that affect how much of the rain will become run-off the coefficient selection is dependent on land use and soil condition.

Table 1 of Run off Coefficient

| Land use | Run off coefficient C |  |
| --- | --- | --- |
|  | For Minor system (<=10 year (ARI) | For Major system (>10 years ARI) |
| Residential Bungalow Semidetached Bungalow Link and terrace house Flat and apartment Condominium | 0.65 0.70 0.80 0.80 0.75 | 0.70 0.75 0.90 0.85 0.80 |
| Commercial and business centers | 0.90 | 0.95 |
| Industrial | 0.90 | 0.95 |
| Sport Field, Parks and Agriculture | 0.30 | 0.40 |
| Open spaces Bare soils (no cover Grass cover Bush cover Forest cover | 0.50 0.40 0.35 0.30 | 0.60 0.50 0.45 0.40 |
| Roads and Highways | 0.95 | 0.95 |
| Water body(pond) Detention pond (with outlet) Renton pond (no outlet) | 0.95 0.00 | 0.95 0.00 |

Note: The Runoff coefficients in the table above are given as a guide for designers. The near field runoff                                                                                   coefficient for any single or mixed land use should be determined based on the imperviousness of the   area

### 5.3.1 Rainfall intensity, mm/hr

It is the rate of rainfall over an interval of time such that the intensity multiplied by duration equals to amount of rain.

Using Intensity Duration Frequency Curve to determine rainfall intensity

The Intensity Duration Frequency (IDF) Curves are usually used to determine the rainfall intensity of a selected catchment area. The IDF curves method calculates the 2, 5, 10, 25, 50 and 100- year return intervals for each duration period and requires the reading of the value from the graphs provided according to the specified design return period.

From the rainfall intensity- duration frequency curve from Atlas for Kenya intensity 12hr storm for a 10-yr flood was found to be 5.8mm/hr.

The catchment is characterized by tarmac road in the airport and A2 road, commercial center and connecting earth roads and expansive airport grassland.

Q = CIA/3.6

= (0.95*0.17+1.75*0.4+0.2*0.9)*5.8*/3.6

= 1.69m3/s

| Sub catchment | Area, A, Km2 | Coefficient, C Dimensionless | Q = CIA/3.6 (m3/s) |
| --- | --- | --- | --- |
| Tarmac Area | 0.17 | 0.95 | 0.26 |
| Commercial buildings | 0.2 | 0.9 | 0.29 |
| Grassland/all-weather roads | 1.75 | 0.45 | 1.127 |
| Total | 2.12 |  | 1.690 |

## 5.4 	Water conveyance channel design

The design of a conveyance system involves the two major steps:

- Selection of the type and layout of the system types and alignment of the canals, type and location of the structure etc.

- Determination of the hydraulic dimensions of different components of the systems (Hydraulic design).

### 5.4.1 Determining geometric dimension of the channel

The most efficient cross section, carrying the highest flow per unit cross section area, of an open channel is a semi-circle. This is because it has a minimum wetted perimeter and a maximum hydraulic radius. Semi-circle cross sections however difficult to construct and maintain and therefore the trapezoidal channels are commonly used. (Sleigh, 2007) the catch pit grates and entrance screens will be placed on top of the channel for pollutant removal.

By using manning’s formula, the sizing of the channel is determined:

Q = AV

V = 1/nR(2/3)So(1/2)

Where,	Q = Discharge, m3/s

A = Area, m2

V = Velocity of flow, m/s

N = manning’s coefficient, 0.015 (rough concrete)

So = Slope, 0.015

R = Hydraulic radius, A/P

Where, n is concrete roughness for finished float concrete.

Side Slopes of the trapezoidal channel

The best design of a trapezoidal cross section is the one with a bottom half of a hexagon, such that its side slopes are inclined 60 degrees from the horizontal (Sleigh, 2007). Steep side slopes are desirable in that they save on land loss. However, they pose the danger of bank caving in if the banks are not protected and the slope is greater or equal to the soil’s angle of repose.

### 5.4.2 Determining the best channel through interaction (comparing rectangular and trapezoidal channel)

Trapezoidal Channel

Using Manning equation and through a developed excel sheet the dimensions are per the tables below.

Trapezoidal channel; b = 0.75m, h = 0.9 m, B = 2.55m (including a freeboard of 0.3m)

Rectangular channel

| RECTANGULAR CHANNEL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b | y | a=by | b=a˄5 | c=b+2y | d=c˄2 | e=b/d | f=0.409-e |
| 1.00 | 1.00 | 1.0000 | 1.0000 | 3.0000 | 9.0000 | 0.1111 | 0.2979 |
| 1.00 | 0.50 | 0.5000 | 0.0313 | 2.0000 | 4.0000 | 0.0078 | 0.4012 |
| 1.50 | 0.50 | 0.7500 | 0.2373 | 2.5000 | 6.2500 | 0.0380 | 0.3710 |
| 1.50 | 1.00 | 1.5000 | 7.5938 | 3.5000 | 12.2500 | 0.6199 | -0.2109 |
| 1.50 | 0.70 | 1.0500 | 1.2763 | 2.9000 | 8.4100 | 0.1518 | 0.2572 |
| 1.50 | 0.90 | 1.3500 | 4.4840 | 3.3000 | 10.8900 | 0.4118 | -0.0028 |

Rectangular channel; b = 1.5m, y = 1.2m (including a freeboard of 0.3m)

## 5.5	Conclusion & Recommendation

- It is proposed that a design of a trapezoidal open channel of dimensions: bottom: 0.75m, B= 2.55m, H = 0.9m (including a freeboard of 0.3m) be constructed from Little Angel Academy to Isiolo Girls Secondary (1200m) since the road is wide and there is very minimal traffic.

- It is proposed that a cover rectangular open channel design of dimensions B = 1.5m, H = 1.2m (including freeboard of 0.3m) from Isiolo Girls Secondary to the road connecting to Merire drain (900m) be constructed to its heavy vehicular and pedestrian use.

- It is further proposed that river widening and deepening works (200m) be carried along Merire River to facilitate the drainage of collected water.

- A two-cell box culvert of 2.0 x 2.0 m shall replace the existing cross pipe culvert at Isiolo – Moyale, A2 road.

- A two-cell box culvert of 4.0 x 2.0m shall replace the existing access pipe culvert at Bulapesa Bridge.

# CHAPTER 6: DESIGN OF MERIRE RIVER TRAINING WORKS ON MERIRE RIVER (ISIOLO AIRPORT JN –BULAPESA BRIDGE, 3.1KM)

## 6.1 Introduction

3.1Km river training works have been designed. The bottom width of 5.6m and depth of 1.3m shall be excavated to required depths from Isiolo Airport Junction to Bulapesa Bridge. Below is the situation of the river at different points along its course.

Merire River at Airport Junction.                                                      Merire River at Isiolo Red Cross Branch.

Sedimentation along the river at SDA Isiolo.     Waso Bridge.

Garbage Blocking a Bridge along the River.          Encroachment along the river.

The specific works comprises of:

- Excavation of earthworks to formation level.

- Replacements of access culverts.

- Relocation of existing utilities along the river.

## 6.2 Summary of design output

| Channel | Merire River training |
| --- | --- |
| County | Isiolo |
| Sub-county | Isiolo |
| Length, km | 3.1 |
| Type of channel | Trapezoidal Channel |
| Channel Dimensions | Top width = 8.2m, Bottom width =5.6m, side, 1.3m Varying Height with 0.3m free board |
| Culvert type | Varies |
| Design Catchment, Km2 | 2.12 |
| Discharge, Q, m3/s | 1.69 |
| Flood return Period | 1 in 50 years |

## 6.3 Hydrology

### 6.3.1 Flood Frequency Analysis

The design of diversion works and storm water channels require an estimation of the peak discharge for a given return period. Insufficient capacity of the channel to hold flood water may lead to water overtopping and cause destruction of lives and properties. In this design a return period of 1 in 50 years was used.

The analysis will be based on Richard’s Method

Richard’s method is based on an empirical formula to calculate the “time of concentration” (TC) of the catchment. The time of concentration is the time it takes for the rain fallen on the furthest point of the catchment to reach the river at the point where the peak flow is to be estimated? It takes into account the rainfall pattern and intensity and the catchment.

Run-off factors for Different soil Types

| Catchment Soil Type | K |
| --- | --- |
| Rocky and impermeable | 0.80 to 1.00 |
| Slightly permeable, bare | 0.60 to 0.80 |
| Slightly permeable, partly cultivated or covered with vegetation | 0.40 to 0.60 |
| Cultivated, absorbent soil | 0.30 to 0.40 |
| Sandy bare soil | 0.20 to 0.30 |
| Heavy forest | 0.10 to 0.20 |

### 6.3.2 Characteristics determining its runoff (Kr).

The formula for calculating Tc is as shown in the below and resolved iteratively for different values of TC until the equation balances.

Equation 8-3: rc3(Tc+1)=C.L2Kr.R.S.f(a)

Where:  TC =time of concentration [hours]

L = longest path of the catchment [KM]

C = a coefficient, function of (K, R) which can be obtained from Figure 8-5

Kr = runoff factor, which can be obtained from table 8-7

R = rainfall coefficient, where R = (t+1)tF

t= selected storm duration [hours]. This should approximate the time of                concentration (Tc)

F = Total rainfall [mm] = intensity [mm/hr] ₓstorm duration (t) [hours], for the selected storm duration and frequency. Intensity is obtained from the rainfall intensity maps in Appendix A or from the rainfall frequency Atlas of Kenya (KMS).

S= the average slope of the catchment

F (a) = ratio of the average rainfall intensity (i) to the maximum rainfall intensity (I) over the catchment area, obtained from figure 8-6

A = the area of the catchment [km2].

Coefficient C, Richard’s Method

Rainfall Intensity Factor, Richard’s Method

Once Tc has been found a revised estimated of the average rainfall intensity for the catchment can be obtained using the equation below:

i= Imax. f(a)

Where i =average rainfall intensity (mm/hr) over the catchment

IMAX =maximum rainfall intensity = RTc+1(mm/hr)

f(a) = Ratio of the average rainfall intensity (i) to the maximum

Rainfall intensity (Imax) over the catchment area, obtained from the Rainfall Intensity Factor, Richard’s Method figure.

Finally, the rational formula is used to calculate the expected maximum flow:

The equation is as shown below:

Qp =Kr i a3.6

Where: QP = peak flow (m3/s) with return period equal to that of the selected storm.

I = average rainfall intensity over the catchment (mm3/hr)

a = area of the catchment (km2)

Kr = Runoff factor (Dimensionless)

## 6.4 Design of channel

### 6.4.1 Catchment area

In order to use the rational method, Richard’s formula was used to obtain the time of concentration and finally the value of i which is the average rainfall intensity.

The first step in designing process was to determine the airport channel catchment area. This was obtained by input of GPS Coordinates of the channel starts point into Global Mapper and using the value of the drainage area and the Longest path (length) as indicated below: channel start GPS Coordinates: 0019’35” N: 37034’27” E

Isiolo County River Basin

## 6.5 Access Culverts

Culverts shall be provided for every access that is used as a passage way by the pedestrians and vehicular traffic.

Typical culverts of (5*2.5) m cross-sectional area shall be replaced in the following areas

- 00 20’02” N 37034’27” E

- 00 20’14” N 37034’37” E

- 00 20’19” N 37034’39” E

- 00 20’28” N 37034’44” E

- 00 20’44” N 37034’44” E

## 6.6 Conclusion and Recommendation

R. Merire has been encroached and some households have settled so close to the river. The natural width of the river seemed to have been approximately 15m. Some sections were observed to be as narrow as 2 to 3 m. This has contributed to the flooding situation in Isiolo as water his hindered from its natural flow of water.

Secondly the community lacks proper solid waste disposal systems and it was observed that lots of dumping of solid waste seemed to be happening at the river.

Isiolo town is like the collection basin, being a low land of the upstream drainage areas like Meru. Consequently, water flows at very high velocity and little rains of the upstream generally leads to floods in Isiolo town.

Isiolo town does not have developed storm water drain system and are in the process of design of the systems that can easily evacuate water from the town.

River Merire was proposed to be deepened rather than widening because of the encroachment challenges. Therefore, for the very narrow areas the county government will need to enforce relocation of people who have settled along the river.

Its proposed that river training works be done along the designed section being 5.6m for the river bottom. 8m at top and 1.5m average height. The access along the river valley access hall be replaced with box culverts.

# CHAPTER 7: DESIGN OF THE ISIOLO AIRPORT JN- KIITHE ROAD JN STORM WATER DRAIN

## 7.1 Introduction

The scope of the design works

Concrete rectangular storm drainage channel along Nairobi-Isiolo-Moyale A2 road from Mwangaza estate to the designed Kiithe Line -2.1km. The drainage channel is also connected to a short line from the tarmac to Bula (Car Wash) draining to Merire river- 200m.

This will be a 2.1Km rectangular channel with bottom width 1.5m, height 1.0m has been design on the right hand side of the Isiolo-Moyale A2 road from Isiolo Airport road turn-off to the Kiithe road.

The proposed construction woks will comprise of

- Excavation of earthwork to formation.

- Construction of a rectangular channel Stone pitching sidewalk.

- Replacements of access culverts.

- Relocation of street lightings and other existing utilities.

## 7.2 Summary of design output

| Channel | Isiolo Airport JN - Kiithe Road JN Storm Water Drain |
| --- | --- |
| County | Isiolo |
| Sub-county | Isiolo |
| Length, km | 2.1 |
| Type of channel | Rectangular Covered Channel |
| Channel Dimensions | Top width = 1.5m, Bottom width =1.5m, side, 1m Varying Height with 0.3m free board |
| Culvert type | Varies |
| Design Catchment, Km2 | 2.12 |
| Discharge, Q, m3/s | 1.69 |
| Flood return Period | 1 in 50 years |

## 7.3 Hydrology

Rational method is used in the determination of design flood in accordance to the consequences of the flood return period of 50 years. The design flood is used in the sizing of the system. Due to heavy motorist and safety of pedestrian on the proposed route, a covered open channel system is proposed along the route.

The Rational method is based on the assumptions that;

- The predicted peak discharge has the same probability of occurrence (return period) as the rainfall intensity.

- The runoff coefficient is constant during the rain storm.

The formula was used to determine the design run-off discharge that shall be carried by the proposed drain.

Q=CIA/3.6=1(∑CA)/3.6

Where Q= peak run-off rate, dimensionless

I= rainfall intensity, mm/hr

A= drainage area, Km

Run-off coefficient, dimensionless

Dimensionless value representing characteristics of the watershed that affect how much of the rain will become run-off the coefficient selection is dependent on land use and soil condition.

| Surface Type | Runoff coefficient (%) |
| --- | --- |
| Roof tiles, corrugated sheets, plastic sheets, concreted bitumen. | 80 |
| Brick pavement | 60 |
| Compacted soil | 50 |
| Uncovered surface, flat terrain | 30 |
| Uncovered surface, 0-5% | 40 |
| Uncovered slope, 0-10% | 50 |
| Uncovered slope >1% | >50 |

Intensity (mm/hr) for a flood of 50 years return period

Using Isohyetal mean annual rainfall of 1981 to 2010

The isohyetal maps are used to determine the rainfall intensity of selected catchment area. The maps calculates the 2, 5,10, 25, 50, and 100- year return intervals for each duration periods and provides the rainfall intensity according to the specified design return period.

The rainfall intensity for Isiolo for a 50-yr flood 12 hr storm is 10mm/hr.

Drainage Catchment Area

Q=CIA/3.6=1(∑CA)/3.6

The drainage catchment is approximately 2Km2 (Google Earth) and is characterised with galvanised iron sheet roofed houses (30%) developed in plots, uncovered surface slope (0-5%)-65% and a tarmac road (5%).

Therefore;

Tarmac area = 0.05*2=0.1km2

Housing area = 0.3*2=0.6Km2

Uncovered surface slope (0-5%) -(0.6+0.1) =1.3Km2

Discharge, Q

| Catchment Characteristic | Area (km2), A | Run-off Coefficient, C | Intensity (mm/hr), I | Q,m3/s |
| --- | --- | --- | --- | --- |
| Tarmac | 0.1 | 0.8 | 10 | 0.22 |
| Housing | 0.6 | 0.8 | 10 | 1.33 |
| Uncovered surface slope (0-5%) | 1.3 | 0.4 | 10 | 1.44 |
|  |  |  |  | 2.99 |

## 7.4 Hydraulic Design

The geometric design of the proposed channel.

Sizing the channel is done using Manning’s formula. The rectangular channel is chosen to discharge the storm water drain.

Table of equations for rectangular trapezoidal and circular channels.

Q= AV

V=1/nR(2/3)S0(1/2)

Where, Q=Discharge, m 3/s

A= Area, m2

V=Velocity of flow, m/s

N= Manning’s n coefficient for concrete is 0.02

S0= Slope, (H/L), Where 	 H, (1179-1113066)

L, Longest distance between the two points on a straight path, 2.4Km

S0=66/2400=0.0275

R= Hydraulic radius, A/P

Q=1/nA^ (5/3)/P^ (2/3) S0^ (1/2)

2.99=10.02*by53b+2y2/3*0.002751/2

0.0469=by5[b+2y)]2

Using iteration to determine the size of b and y

Bottom width b=1.5m, water depth, y=0.6m, freeboard=0.3m

For protection of the drain, on the left-hand side stone – pitching shall be placed connecting to the road shoulders and the left-hand side shall be grassed.

Due to the general depth of the storm drain, it shall be covered with a 50mm thick slotted concrete covers to allow surface run-off to flow in and for public safety. The covers shall also help against dumping into the drains. However, they shall be constructed in a manner such that the can be easily removed for the cleaning of the drains.

Determination of cut

Average cut volume= 2,100*1.1*1.5= 3465M3

## 7.5 Design of Culverts

Culverts are designed//provided for through road accesses from the main road and road crossing.

For geometric design of the culverts; manning’s formula is used as below:

Q=AV

V 01/nR (2/3) S0(1/2)

Assume a V =1m/s

Q = AV

- Assessing the adequacy of the 4 no 0.9m Dia cross culverts situated 0019’49” N

37034’28” E upstream oilbya petrol station to discharging Q=2.99m/s independently.

2.99=3πd24

D=0.98m

The existing culverts can only discharge Q of 2.54m3(85%) independently.

However, since the discharge is split between the drain channel and the cross culverts, it shall be assumed that they discharge 35% of Q and the drain continues to carry 65%Q.

- To provide for access culverts toward Mwanganza Estate before Oilbya petrol station.

It shall be assumed that they shall drain a 0.65Q, 1.9435M3/s. a trial sizing is made by using 3 No pipe culverts for draining the discharge:

1.9435 =3A *1

1.9435= 3πd24

D=0.91m

Therefore provide 4 No. 0.9m dia access culverts at Mwanganza Estate access.

### SCHEDULE OF NEW CULVERTS AND THOSE TO BE REHABILITATED

- 0019’56” N37034’32” E – 4 No. 0.9m Dia (NEW)

- 0020’10” N 37034’41” E – 3 No. 0.9m Dia (REHABILITATION)

- 0020’10” N 37034’41” E – 3 No. 0.9m Dia (REHABILITATION)

- 0020’10” N 37034’41” E – 3 No. 0.9m Dia (REHABILITATION)

- 0020’13” N 37034’44” E – 3 No. 0.9m Dia (NEW SHELL PETROL STN)

- 0020’14” N 37034’47” E – 3 No. 0.9m Dia (NEW ACCESS TO LITTLE ANGELS)

- 0020’19” N 37034’48” E– 3 No. 0.9m Dia (REHABILITATION)

- 0020’27” N 37034’48” E– 3 No. 0.9m Dia (REHABILITATION)

- 0020’33” N 37034’48” E – 3 No. 0.9m Dia (REHABILITATION)

- 0020’41” N 37034’51” E – 3 No. 0.9m Dia (REHABILITATION- ISIOLO BOYS)

- 0020’43” N 37034’51” E – 3 No. 0.9m Dia (REHABILITATION - STADIUM)

- 0020’45” N 37034’51” E – 3 No. 0.9m Dia (REHABILITATION)

- 0020’47” N 37034’51” E – 3 No. 0.9m Dia (REHABILITATION)

- 0020’50” N 37034’52” E – 3 No. 0.9m Dia (REHABILITATION)- CROSS CULVERT

- 0020’52” N 37034’53” E – 3 No. 0.9m Dia (REHABILITATION)

## 7.6 Conclusion and Recommendations

- It is proposed to divert a discharge of 2.99m3/s mostly from Mwanganza estate a concrete rectangular storm drain to be constructed along the Isiolo Moyale road to carry and divert water to Merire river.

- It’s further recommended for replacement of access culvert that were constructed long time ago be replaced with culverts that can be able to withstand the discharge.

- Protection works for the drains are like grassing and stone pitching are to be constructed to protect the drain.

# CHAPTER 8: DESIGN OF NOTHERN SIDE OF ISIOLO AIRPORT TO 78 BATALLION STORM WATER DRAIN CHANNEL

## 8.1 Introduction

The channels from the Airport to 78 Battalion is trapezoidal earth channel bottom width 3.4m and top width 6m and 2.83Km long. The works comprises of but not limited to:

- Excavation of the channel.

- Rehabilitation of the existing gabions at the Airport.

- Construction of concrete sills along the channel to prevent channel erosion.

- Extension of the channel inside the airport to act as a cut-off drain and divert some floods from the airport.

- Construction of precast pipe culverts 900mm in diameter at specific locations along the channel.

## 8.2 Summary of design output

| Channel | Airport Channel to 78 Battalion |
| --- | --- |
| County | Isiolo |
| Sub-county | Isiolo |
| Length, km | 2.83 |
| Type of channel | Trapezoidal Channel |
| Channel Dimensions | Top width = 6m, Bottom width =4m, side slopes 1:1, 1m Varying Height with 0.3m free board |
| Culvert type | Triple pipe precast culvert each of Diameter 0.9m |
| Design Catchment, Km2 | 5.03 |
| Discharge, Q, m3/s | 19.61 |
| Flood return Period | 1 in 50 years |

## 8.4 Design of the Channel.

Richard’s Method was used to calculate the “time of concentration” (Tc) of the catchment.

Once Tc had been found the rational formula was finally used to calculate the expected

Maximum flow (Peak).

### 8.4.1 Catchment area

The first step in the design process was to determine the airport channel catchment area. This was obtained by input of GPS Coordinates of the channel start point into Global Mapper and using the values of the drainage area and the Longest (length) as indicated below:

Airport Channel starts GPS Coordinate: Northing: 0.337920 Easting: 37.592780

Catchment area = 5.03km2

Total catchment area used was an approximate value of 5.03km2. Length of the longest path = 5.248km.

### 8.4.2 Determination of TC, QP and channel sizing

| TC3/(Tc+1) =C.L2/[Kr.R.S.f(a)] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tc – time of concentration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C – coeff |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L – longest path of catchment km |  |  |  | 5.248 |  |  | km |  |  |  |  |  |  |  |  |  |
| Kr =0.5 from table 8-7 |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |
| f(a) – 0.78 fig 8-7 |  |  |  | 0.78 |  |  |  |  |  |  |  |  |  |  |  |  |
| i = I.f(a) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i = average rainfall intensity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I = maximum rainfall intensity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F =Tot rainfall in mm for storm from Appendix A (intensity maps) |  |  |  | 10 |  | Mm/hr |  |  | t= | 12 |  | hr |  | 120 |  | F |
| R = [(t+1)/t]F |  |  |  | 130 |  |  |  |  |  |  |  |  |  |  |  |  |
| t =12hrs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Therefore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R = | 130 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | H1 |  |  | H2 |  |  | dH |  | S |  |  |  |
| S =Slope of catchment = h/L |  |  |  |  | 1163.2 |  |  | 1058.3 |  |  | 104.9 |  | 0.02 |  |  |  |
| dH | 104.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L (m) | 5248 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Slope | 0.001999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kr =0.5 | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kr.R | 65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C = f(Kr.R) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| f( 65 )= |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fig. 8-5 | 0.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L = ……km 5.248 km |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tc3/(Tc+1) =C.L2/[Kr.R.S.f(a)=G 3.805 = Tc3/(Tc+1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solve =Tc3- G*Tc – G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tc = | 2.612 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I = R/(Tc+1) = | 35.991 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i = I.f(a)= | 28.073 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a (area of catchment) km2 | 5.03 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Qpeak =Kr.i.a/3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Qpeak 19.61 m3/s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

From the calculated Q the channel dimensions are calculated to bottom width 4m Top width 6m and depth of 1 m

with a freeboard of 0.3m

## 8.4 Determination of volumes of cut and fill.

| Chainage | Fill | Cut | Average area (Cut) | Average Area (Fill) | Dist (m) | d vol Cut (m3) | d vol Fill (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 3.60 | 4.43 |  |  |  |  |  |
| 100.00 | 8.82 | 5.68 | 5.06 | 6.21 | 100.00 | 505.50 | 621.00 |
| 200.00 | 5.89 | 4.69 | 5.19 | 7.36 | 100.00 | 518.50 | 735.50 |
| 300.00 | 0.00 | 10.32 | 7.51 | 2.95 | 100.00 | 750.50 | 294.50 |
| 400.00 | 1.43 | 4.75 | 7.54 | 0.72 | 100.00 | 753.50 | 71.50 |
| 500.00 | 3.72 | 1.15 | 2.95 | 2.58 | 100.00 | 295.00 | 257.50 |
| 600.00 | 5.32 | 1.55 | 1.35 | 4.52 | 100.00 | 135.00 | 452.00 |
| 700.00 | 4.30 | 0.77 | 1.16 | 4.81 | 100.00 | 116.00 | 481.00 |
| 800.00 | 7.08 | 0.00 | 0.39 | 5.69 | 100.00 | 38.50 | 569.00 |
| 900.00 | 4.39 | 0.00 | 0.00 | 5.74 | 100.00 | 0.00 | 573.50 |
| 1000.00 | 0.44 | 7.44 | 3.72 | 2.42 | 100.00 | 372.00 | 241.50 |
| 1100.00 | 2.35 | 3.78 | 5.61 | 1.40 | 100.00 | 561.00 | 139.50 |
| 1200.00 | 3.27 | 1.26 | 2.52 | 2.81 | 100.00 | 252.00 | 281.00 |
| 1300.00 | 3.51 | 3.02 | 2.14 | 3.39 | 100.00 | 214.00 | 339.00 |
| 1400.00 | 3.84 | 1.98 | 2.50 | 3.68 | 100.00 | 250.00 | 367.50 |
| 1500.00 | 1.39 | 3.61 | 2.80 | 2.62 | 100.00 | 279.50 | 261.50 |
| 1600.00 | 0.37 | 7.31 | 5.46 | 0.88 | 100.00 | 546.00 | 88.00 |
| 1700.00 | 8.72 | 2.46 | 4.89 | 4.55 | 100.00 | 488.50 | 454.50 |
| 1800.00 | 2.83 | 7.12 | 4.79 | 5.78 | 100.00 | 479.00 | 577.50 |
| 1900.00 | 4.64 | 2.39 | 4.76 | 3.74 | 100.00 | 475.50 | 373.50 |
| 2000.00 | 3.16 | 8.15 | 5.27 | 3.90 | 100.00 | 527.00 | 390.00 |
| 2100.00 | 0 | 7.76 | 7.96 | 1.58 | 100.00 | 795.50 | 158.00 |
| 2200.00 | 0.58 | 8.96 | 8.36 | 0.29 | 100.00 | 836.00 | 29.00 |
| 2300.00 | 4.68 | 0.83 | 4.90 | 2.63 | 100.00 | 489.50 | 263.00 |
| 2400.00 | 3.25 | 5.92 | 3.38 | 3.97 | 100.00 | 337.50 | 396.50 |
| 2500.00 | 3.93 | 2.27 | 4.10 | 3.59 | 100.00 | 409.50 | 359.00 |
| 2600.00 | 0.84 | 7.56 | 4.92 | 2.39 | 100.00 | 491.50 | 238.50 |
| 2700.00 | 1.35 | 7.45 | 7.51 | 1.10 | 100.00 | 750.50 | 109.50 |
| 2800.00 | 0 | 3.99 | 5.72 | 0.68 | 100.00 | 572.00 | 67.50 |
| 2833.95 | 0 | 5.22 | 4.61 | 0.00 | 33.95 | 156.34 | 0.00 |
| TOTAL |  |  |  |  | 2833.95 | 12395.34 | 9190.00 |

## 8.5 Conclusion and Recommendation

- It’s proposed that a channel of top dimension 6m and bottom dimension 4 m should be constructed to divert floods.

- The channel should have sill protection at 50m interval to prevent erosion of the channel since lining of the whole channel will be very costly.

- Periodic maintenance of the channel by the county is recommended due to effects of siltation which causes clogging of the culverts especially towards 78 battalion.

- Rehabilitation of the existing gabions near the airport.

- The drain to be extended inside the airport to act as cut-off drain to minimize the floods that flow the airport to Isiolo town.

# CHAPTER 9:  DESIGN OF NGARAMERA FLOOD CONTROL WORKS & NGAREMARA “A”, “B” AND “C’.

## 9.1 Project data

| Project | - Ngaremara “A” Gabions - Ngaremara “B” - Ngaremara “C” No work to be done - Ngaremare “D” Channel and Dyke |
| --- | --- |
| County | Isiolo |
| Sub – county | Isiolo |
| Gabion details | Gabion Box 1mX1mX1m Gabion Mattress 1mX0.3mX2m |
| Channel details |  |
| Length, km | 0.92 |
| Type of channel | Trapezoidal Channel |
| Channel Dimensions | Top width = 5.5m, Bottom Width = 2.9m, Side slope 1:1, 1m Varying Height with 0.3m free board. |
| Dyke Details |  |
| Embankment top width (m) | 5.5 |
| Embankment bottom width (m) | 2.9 |
| Side slopes | 1:1 |
| Embankment length (km) | 0.3 |
| Design Catchment, Km2 | Ngaremara “A”, “B” and “C” = 28.47 Ngaremara “D” |
| Discharge, Q, m3/s | Ngaremara “A” = 26.19 Ngaremara “B” = 25.76 Ngaremara “C” = 25.33 Ngaremara “D” = 10.19 |
| Flood return period | 1 in 50 years |

## 9.2 Hydraulics

Richard’s method was used to calculate the “time of concentration” (TC) of the catchment.

Once TC had been found the rational formula was finally used to calculate the expected

Maximum formula flow (Qpeak)

## 9.3 Catchment area

The first step in the design process was to determine the channel catchment area. This was obtained by input of GPS Coordinates of the channel start point into Global Mapper and using the values of the drainage area and the longest path (length).

### 9.3.1 Determination of TC, Qp and Q flowing in each Culvert.

| Peak flow calculations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC3/(TC+1) C.L2/[Kr.R.S.f(a)] |  |  |  |  |  |  |  |  |  |
| TC – time of concentration |  |  |  |  |  |  |  |  |  |
| C – Coeff = |  |  |  |  |  |  |  |  |  |
| L – longest path of catchment km 20.26km |  |  |  |  |  |  |  |  |  |
| Kr=0.5 from table 8-7 | 0.5 |  |  |  |  |  |  |  |  |
| F (a) – 0.78 fig 8-6 | 0.73 |  |  |  |  |  |  |  |  |
| i = average rainfall intensity |  |  |  |  |  |  |  |  |  |
| I = maximum rainfall intensity |  |  |  |  |  |  |  |  |  |
| F =Tot rainfall in mm for Storm from Appendix A (intensity maps) 10 mm/hr = t 12hr 120 F |  |  |  |  |  |  |  |  |  |
| R = [(t+1)/t]F 130 |  |  |  |  |  |  |  |  |  |
| t = 12hrs |  |  |  |  |  |  |  |  |  |
| Therefore |  |  |  |  |  |  |  |  |  |
| R = | 130 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | H1 | H2 | dH |  | S |
| S = slope of catchment h/L |  |  |  |  | 1095.16 | 946.67 | 148.49 |  | 0.0073 |
| dH | 148.49 |  |  |  |  |  |  |  |  |
| L(m) |  | 20260 |  |  |  |  |  |  |  |
| slope |  |  |  |  |  |  |  |  |  |  |
| Kr =0.5 |  |  |  |  |  |  |  |  |  |  |
| Kr.R |  |  |  |  |  |  |  |  |  |  |
| C= f(Kr.R) |  |  |  |  |  |  |  |  |  |  |
| f (65) = |  |  |  |  |  |  |  |  |  |  |
| Fig 8-5 |  | 0.14 |  |  |  |  |  |  |  |  |
| L =……km 20.26km |  |  |  |  |  |  |  |  |  |  |
| TC3/(TC+1) = C.L^2/[Kr.R.S.f(a)]=G 165.235 =TC3/(TC+1) |  |  |  |  |  |  |  |  |  |  |
| Solve =TC^3-G*TC-G |  |  |  |  |  |  |  |  |  |  |
| TC |  | 13.328 |  |  |  |  |  |  |  |  |
| I =R/(TC+1) = |  | 9.073 |  |  |  |  |  |  |  |  |
| i =I.f(a)= |  | 6.623 |  |  |  |  |  |  |  |  |
| a (area of catchment) km2 |  | 28.47 |  |  |  |  |  |  |  |  |
| QPeak Kr.i.a/3.6 |  |  |  |  |  |  |  |  |  |  |
| Qpeak 26.19m3s |  |  |  |  |  |  |  |  |  |  |
| From “practice Manual of small dams and pans and other water Conservation Structures |  |  |  |  |  |  |  |  |  |  |
| In Kenya second edition |  |  |  |  |  |  |  |  |  |  |
| Flow in culvert A |  |  |  |  |  |  |  |  |  |  |
| Diameter of culvert=0.9mm Assuming ¾ full flow during floods |  |  |  |  |  |  |  |  |  |  |
| Adopt a Velocity of water flow =0.9m/s as water flows in |  |  |  |  |  |  |  |  |  |  |
| Culvert A |  |  |  |  |  |  |  |  |  |  |
| Q in Culvert A = 0.9m/s × 0.48m2 = 0.43m3/s ……. Flow in culvert A |  |  |  |  |  |  |  |  |  |  |
| Flow in culvert B |  |  |  |  |  |  |  |  |  |  |
| Flow left after spill over in culvert A = 26.19 – 0.43 = 25.76m3/s |  |  |  |  |  |  |  |  |  |  |
| Diameter of culvert = 0.9mm Assuming ¾ full flow during floods |  |  |  |  |  |  |  |  |  |  |
| Area = 3.142 × 0.45×0.45×0.75 =0.48m2 |  |  |  |  |  |  |  |  |  |  |
| Adopt a velocity of water flow = 0.9m/s as water flows in In culvert B |  |  |  |  |  |  |  |  |  |  |
| Q in culvert B = 0.9m/s×0.48m/s = 0.43m3/s …………Flow in culvert B |  |  |  |  |  |  |  |  |  |  |
| Flow in culvert C (Twin pipe Culvert) |  |  |  |  |  |  |  |  |  |  |
| Flow left after spill over in culvert B = 25.76-0.43 = 25.33m3/s |  |  |  |  |  |  |  |  |  |  |
| Diameter of culvert = 0.9mm Assuming ¾ full flow during floods |  |  |  |  |  |  |  |  |  |  |
| Area = 3.142*0.45*0.45*0.75 =0.48m2*2 = 0.96m2 |  |  |  |  |  |  |  |  |  |  |
| Adopt a velocity of water flow =0.9m/s as water flows through the twin culvert B |  |  |  |  |  |  |  |  |  |  |
| Q in Twin Culvert B = 0.9m/s* 0.96m2 =0.86m3/s……. Flow in culvert C. N/B The culvert capacity is not sufficient to evade the floods waters. |  |  |  |  |  |  |  |  |  |  |

## 9.4 Conclusion and Recommendations

- It is proposed that gabions to be constructed on Ngaremara “A” and “B” site to hold the floods water and allow it spill over to the respective culverts.

- Rehabilitation of the existing gabions is also recommended

- The existing Culverts do not have enough capacity to evade the floods waters it is therefore recommended in the future the county should come up with design and construction of appropriate culvert size.

# CHAPTER 10: DESIGN OF NGAREMARA ‘D”

## 10.1 Catchment area

Ngaremara survey start GPS Coordinates:  Northing :0.48720   Easting: 37.65060

Catchment area = 5.03km2

Total catchment area used was an approximate value of 5.03km2. Length of the longest path = 5.248km.

## 10.2 Determination of TC, QP and Channel sizing

|  | Peak flow Calculations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TC3/(TC+1) = C.L2/[Kr.R.S.f(a)] |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TC – time of Concentration |  |  |  |  |  |  |  |  |  |  |  |  |
|  | C – coeff = |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L = longest path of Catchment 18.29 km |  |  |  |  |  |  |  |  |  |  |  |  |
| Kr =0.25 from table 8-7 |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |
| f(a) – 0.78 fig 8-6 |  |  | 0.713 |  |  |  |  |  |  |  |  |  |  |
| i = I.f(a) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i = average rainfall intensity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I = maximum rainfall Intensity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F = Tot rainfall in mm For storm from Appendix A (Intensity maps) 10 mm/hr t=12 hr 120 F |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R= [(t+1)/t) F 130 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| t =hrs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Therefore |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R = |  | 130 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | H1 |  | H2 | dH |  | S |
| S= slope of Catchment =h/L |  |  |  |  |  |  |  | 1086.3 |  | 949.096 | 137.154 |  | 0.0075 |
| dH |  | 137.154 |  |  |  |  |  |  |  |  |  |  |  |
| L (m) |  | 18290 |  |  |  |  |  |  |  |  |  |  |  |
| Slope |  | 0.00750 |  |  |  |  |  |  |  |  |  |  |  |
| Kr=0.25 |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |
| Kr.R |  | 32.5 |  |  |  |  |  |  |  |  |  |  |  |
| C =f(Kr.R) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| f()= |  | 32.2 |  |  |  |  |  |  |  |  |  |  |  |
| Fig 8-7 |  | 0.2 |  |  |  |  |  |  |  |  |  |  |  |
| L = …….km 18.29 km |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TC3/(TC+1) = C.L2/[Kr.R.S.f(a) =G 385.026 =TC3/(TC+1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solve = Tc ^3-G*TC-G |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TC |  |  |  | 20.104 |  |  |  |  |  |  |  |  |  |
| I = R/(TC+1) = |  |  |  | 6.160 |  |  |  |  |  |  |  |  |  |
| i =I.f(a) |  |  |  | 4.392 |  |  |  |  |  |  |  |  |  |
| a (area of catchment) km2 |  |  |  | 33.397 |  |  |  |  |  |  |  |  |  |
| Qpeak = Kr.i.a/3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Qpeak 10.1862 m3s |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Trapezoidal channel |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q = A. R^ (2/3). i^ (.5)/n |  |  |  |  |  |  |  |  |  |  |  |  |  |
| h |  |  |  | 1 |  |  | Meters trial |  |  |  |  |  |  |
| A = 4.9h-(h)^2 |  |  |  | 3.9 |  |  |  |  |  |  |  |  |  |
| Pwet 4.9+ 0.828h |  |  |  | 5.728 |  |  |  |  |  |  |  |  |  |
| R = A/Pwet |  |  |  | 0.6809 |  |  |  |  |  |  |  |  |  |
| n= .025 |  |  |  | Earth |  |  |  |  |  |  |  |  |  |
| n =.013 |  |  |  | Concrete |  |  |  |  |  |  |  |  |  |
| Therefore n= |  |  |  | 0.025 |  |  |  |  |  |  |  |  |  |
| i (channel slope) |  |  |  | 0.0075 |  |  |  |  |  |  |  |  |  |
| Q= A. R^ (2/3). I^ (.5)/n 10.4551 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| But should be up = 10.1862 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Difference -0.26892 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Trial and error h = 1 m |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From “period of manual of small dams and pans and other water conservation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Structures in Kenya, 2nd edition” |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 10.3 Determination of volumes of cut and fill.

| Chainage | Fill | Cut | Average Area (cut) | Average Area (Fill) | Dist (m3) | d vol Cut (m3) | D vol fill(m3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 12.35 |  |  |  |  |  |  |
| 50.00 | 12.12 |  |  | 12.24 | 50.00 |  | 611.75 |
| 100.00 | 12.00 |  |  | 12.06 | 50.00 |  | 603.00 |
| 150.00 | 12.36 |  |  | 12.18 | 50.00 |  | 609.00 |
| 200.00 | 12.06 |  |  | 12.21 | 50.00 |  | 610.50 |
| 250.00 | 12.15 |  |  | 12.11 | 50.00 |  | 605.25 |
| 300.00 | 7.80 | 2.40 |  | 9.98 | 50.00 | 120.00 | 498.75 |
| 350.00 |  | 6.13 | 4.265 | 3.90 | 50.00 | 306.50 | 195.00 |
| 400.00 |  | 6.14 | 6.315 |  | 50.00 | 307.00 |  |
| 450.00 |  | 5.89 | 6.015 |  | 50.00 | 294.50 |  |
| 500.00 |  | 5.80 | 5.845 |  | 50.00 | 290.00 |  |
| 550.00 |  | 6.00 | 5.9 |  | 50.00 | 300.00 |  |
| 600.00 |  | 6.83 | 6.415 |  | 50.00 | 341.50 |  |
| 650.00 |  | 10.21 | 8.52 |  | 50.00 | 510.50 |  |
| 700.00 |  | 9.93 | 10.07 |  | 50.00 | 496.50 |  |
| 750.00 |  | 8.91 | 9.42 |  | 50.00 | 445.50 |  |
| 800.00 |  | 13.11 | 11.01 |  | 50.00 | 655.50 |  |
| 850.00 |  | 13.67 | 13.39 |  | 50.00 | 683.50 |  |
| 900.00 |  | 8.99 | 11.33 |  | 50.00 | 449.50 |  |
| 950.50 |  | 7.76 | 8.375 |  | 50.00 | 388.00 |  |
| 1000.00 |  | 8.74 | 8.25 |  | 50.00 | 437.00 |  |
| 1050.00 |  | 9.25 | 8.995 |  | 50.00 | 462.50 |  |
| 1100.00 |  | 8.40 | 8.825 |  | 50.00 | 420.00 |  |
| 1150.00 |  | 8.32 | 8.36 |  | 50.00 | 416.50 |  |
| 1200.00 |  | 7.45 | 7.885 |  | 50.00 | 372.00 |  |
| 1218.62 |  | 0.00 | 3.725 |  | 18.62 | 0.00 |  |
| TOTAL |  |  |  |  | 1218.62 | 7696.50 | 3733.25 |

## 10.4 Conclusion and Recommendations

- It is proposed that a channel t top dimensions 5.5 m and bottom dimensions of 2.9m should be constructed to divert floods.

- It is also proposed that a section of the works should have a dyke and to protect the slopes of the dyke from erosion, grass should be planted.

- Periodic maintenance of the channel by the county is recommended

References

National Water Master Plan 2030

Isiolo River Basin Integrated Flood Management Plan of 2014 by JICA

County Government of Isiolo Flood Disaster Management Task Force.

County Integrated Development Plan Isiolo County 2018